Ischia Seismic Scenario

Procedure to create an intensity seismic scenario at a local scale of the island of Ischia.

This QGIS® plugin allows calculating seismic scenarios expressed in terms of macroseismic intensity according to deterministic or probabilistic models of attenuation.

The input parameters required to simulate an earthquake, are:

- epicentral location, expressed in degrees (xx.yyy);

- epicentral intensity (I0) or magnitude. In the latter case, the magnitude (Md or MW) is converted into intensity by the relationships obtained by Selva et al. (2021)*.

Selection of the attenuation models:

- deterministic: scenarios for epicentral intensity 6 ≤ I0 ≤ 11 (MCS);

- probabilistic: scenarios for epicentral intensity 8 ≤ I0 ≤ 11 (MCS).

Results are plotted on map with a grid with node distances of 0.250 km. For the probabilistic scenario, the mode of the distribution is taken as the expected intensity at that site.

This QGIS® plugin is a product (supplementary material) related to the paper:

Azzaro R, D'Amico S, Rotondi R, Varini E (2023) The attenuation of macroseismic intensity in the volcanic island of Ischia (Gulf of Naples, Italy): comparison between deterministic and probabilistic models and application to seismic scenarios. Bull Earth Eng, submitted.

*see Table 2 in: Selva J, Azzaro R, Taroni M, Tramelli A, Alessio G, Castellano M, Ciuccarelli C, Cubellis E, Lo Bascio D, Porfido S, Ricciolino P, Rovida A (2021) The seismicity of Ischia Island, Italy: an integrated earthquake catalogue from 8th century BC to 2019 and its statistical properties. Frontiers, Earth Sci 9, 629736